# PUMP INSPECTION RECOMMENDATION REPORT

## Document Information
- **Report Date**: 2026-12-19
- **SOP Reference**: SOP-PUMP-402-REV3, SOP-1042
- **Equipment**: Centrifugal Slurry Pump P-104A / P-104B
- **Facility**: Industrial Operations Unit 4

---

## 1. VIBRATION ANALYSIS LIMITS

| Parameter | Limit Type | Threshold (mm/s RMS Velocity) |
|-----------|------------|-------------------------------|
| Acceptable Operating Limit | Normal Operation | <= 4.5 mm/s |
| Warning Alert Threshold | Monitor Required | 4.6 - 6.0 mm/s |
| Critical Trip Limit | Immediate Shutdown | > 6.0 mm/s |

**Measurement Points**: Horizontal, vertical, and axial planes at DE (Drive End) and NDE (Non-Drive End) bearing housings.

---

## 2. BEARING ASSEMBLY LIMITS

| Parameter | Limit Type | Threshold (°C) |
|-----------|------------|------------------|
| Normal Operating Range | Standard | 50°C - 72°C |
| Maximum Allowable Steady-State | Warning | 75°C |
| High-Temperature Alarm | Alert | 85°C |
| Safety Trip Threshold | Critical | 95°C |

**Lubrication Schedule**: Replenish 25g of ISO VG 220 synthetic polyurea grease every 500 operating hours.

---

## 3. MECHANICAL SEAL CRITERIA

| Parameter | Limit Type | Threshold |
|-----------|------------|----------|
| Plan 32 External Flush Pressure | Minimum Requirement | 15 PSI above stuffing box cavity pressure |
| Seal Face Leakage - Acceptable | Normal Operation | <= 5 drops/minute |
| Seal Face Leakage - Warning | Early Degradation | 6-20 drops/minute |
| Seal Face Leakage - Critical | Overhaul Required | > 20 drops/minute |

---

## 4. INSPECTION PROCEDURE SUMMARY

1. **LOTO Verification** - Confirm power disconnected and lockout/tagout applied
2. **Casing Inspection** - Check for visible cracks, corrosion, or leaks
3. **Oil Level Check** - Verify bearing housing oil between MIN and MAX lines
4. **Shaft Rotation Test** - Rotate by hand to ensure free movement (no binding)
5. **Mechanical Seal Inspection** - Verify intact condition with no excessive dripping
6. **Vibration Measurement** - Document RMS velocity at all measurement points
7. **Temperature Monitoring** - Record bearing housing temperatures

---

## 5. FAILURE CRITERIA & CORRECTIVE ACTIONS

| Condition | Action Required |
|-----------|------------------|
| Low oil level | Do NOT start pump - Report to maintenance immediately |
| Shaft binding during rotation | Full teardown required |
| Vibration > 4.5 mm/s | Schedule vibration spectrum analysis |
| Vibration > 6.0 mm/s | Immediate shutdown and lockout |
| Seal leakage 6-20 drops/min | Monitor - plan seal ring replacement |
| Seal leakage > 20 drops/min | Isolate and overhaul mechanical seal cartridge |
| Bearing temp 85°C+ | Alarm activation - investigate cooling system |
| Bearing temp 95°C+ | Safety trip - immediate shutdown |
| Impeller clearance > 2.5 mm | Mandatory replacement - changeout required |

---

## 6. INSPECTION RECOMMENDATIONS

1. **Pre-Inspection**: Complete LOTO verification and permit documentation
2. **During Inspection**: Measure and document all parameters at current operating levels
3. **Post-Inspection**: Compare readings against limits in this report
4. **Action Items**: 
   - Document any deviations from acceptable limits
   - Schedule maintenance based on severity thresholds
   - Update inspection records with date, inspector name, and measurements
5. **Frequency**: Follow scheduled intervals per facility maintenance program

---

## 7. SAFETY NOTES

- Always verify LOTO before beginning inspection
- Wear appropriate PPE when handling rotating equipment
- Be aware of high pressure in flush systems (15 PSI above cavity pressure)
- Do not start pump with low oil level or binding shaft
- Follow critical trip limits immediately

---

**Report Generated By**: SOVRA Agent  
**Approved For**: Industrial Operations Unit 4 Maintenance Team